Emission Model Agriculture Flanders

EMAV3.0

English Summary

Last update: March 2025

Content

1.	General description EMAV3.0	4

2.	Input data and calculation factors	5

2.1.	Input data checks	5

2.1.1.	Clean-up input data	5

2.1.1.1.	Replace stable types	5

2.1.1.2.	Delete records with empty animal subcategory codes	6

2.1.1.3.	Replace historical animal subcategory codes	6

2.1.1.4.	Replace EA stables	6

2.1.1.5.	Combine stables	7

2.1.2.	Verify input	7

3.	Calculating emissions	10

3.1.	Nitrogen excretion and nitrogen flow	10

3.2.	Volatile solids-excretion	10

3.3.	Section 1: Enteric emissions	11

3.3.1.	Emission stage 'Enteric emissions'	11

3.3.1.1.	Tier 1 methodology	11

3.3.1.2.	Tier 2 methodology	11

3.3.1.3.	Reduction measures Covenant Enteric Emissions Cattle	13

3.4.	Section 2: Manure management	13

3.4.1.	Emission stage 'Pasture'	13

3.4.2.	Emission stages ‘Stable’ and 'Storage'	14

3.4.3.	Emission stage 'Small-scale manure processing'	15

3.4.4.	Emission stage 'Manure processing'	15

3.5.	Section 3: Agricultural soils	16

3.5.1.	Emission stage ‘Pasture’	16

3.5.2.	Emission stage 'Land application'	16

3.5.2.1.	Animal organic manure	16

3.5.2.2.	Synthetic fertiliser	17

3.5.2.3.	Emission calculation	17

3.5.3.	Emission stage ‘Crop residues’	18

3.5.4.	Emission stage 'Soil type'	18

3.5.5.	Indirect nitrous oxide emissions	18

4.	Geographical distribution of emissions	19

5.	Output tables	19

6.	Interpretation of results	19

7.	References	20

# General description EMAV3.0

The Emission Model Agriculture Flanders (EMAV), version 3.0 (2023), calculates the non-energy agricultural emissions of ammonia (NH3), nitrous oxide (N2O), nitrogen oxides (NOx) and methane (CH4,) in Flanders. The EMAV model is divided into 3 main sections. The emission calculations themselves take place in 9 different emission stages, listed below. The whole process is presented schematically in Figure 1.

Figure 1: Schematic representation of the calculation flow in EMAV3.0, divided into nine different emission stages (ES). For each emission stage, the components for which emissions are calculated are shown.

- Section 1 - Enteric emissions:

  - ES Enteric Emissions - emissions caused by digestive processes in farm animals (CH4)

- Section 2 - Manure management:

  - ES Pasture (methane and ammonia) - emissions caused by grazing (CH4, NH3)

  - ES Stable - emissions from the stable (incl. internal manure storage) (CH4, NH3, N2O direct and indirect, NOx)

  - ES Storage - emissions caused by external storage of manure (CH4, NH3, N2O direct and indirect, NOx)

  - ES Small-scale manure treatment - emissions caused by manure treatment at exploitation level (CH4, NH3, N2O direct, NOx)

  - ES Manure processing - emissions caused by (professional) manure processors (NH3, NOx)

- Section 3 - Agricultural soils:

  - ES Pasture (laughing gas and nitrogen oxides) - emissions caused by grazing (N2O direct and indirect and NOx)

  - ES Land application - emissions caused by applying organic manure and fertilisers to plots (NH3, N2O direct and indirect, NOx)

  - ES Crop residues - emissions caused by crop residues left on fields after harvest (NH3, N2O direct and indirect)

  - ES Soil type - emissions caused by mineralisation of soil organic matter and cultivation of organic soils (N2O direct and indirect). The designation 'Soil type' refers to whether the soil is mineral or organic.

# Input data and calculation factors

The input data used by EMAV can be divided into 3 subsets according to origin and level of detail:

- Dataset 1990-1999: Data from the National Institute of Statistics (NIS). The data are at the Flemish level.

- Dataset 2000-2006: The origin of this dataset is the Flemish Land Agency (VLM). These data are also at the Flemish level.

- Dataset 2007-recent: Since 2007, detailed data from the Manure Bank are available. These data cover the whole of Flanders and include, for each exploitation, the number of animals and gross nitrogen production per animal subcategory, the available stable types, the cultivated plots, the crops, etc.

For each year, a separate set of calculation factors is included in the model. Each set consists of several thematically grouped tables. The tables include emission factors, usage shares, correction factors, links between crop codes and crop groups, etc.

The content of the tables can be modified by the user. The user can also create new sets of calculation factors. With each emission calculation, the selected calculation factor set is written to an excel file in the same folder where the emission calculation results are written.

Compared to EMAV version 2.1, the links were revised between the animal categories as used in the input dataset and the stable emission factors for ammonia included in the calculation factors.

## Input data checks

### Clean-up input data

Listed below are the various actions that happen consecutively when loading a new dataset.

### Replace stable types

To obtain uniformity in the time series, certain old names for stable systems are replaced by newer names in this step (see Table 1). Also, missing stable types are completed. This is done either based on the given percentage of liquid manure or on the default housing type assigned for this animal subcategory. The decision rules used for this are shown in Figure 2. This methodology was adopted from EMAV version 2.1.

Table 1 : Old designations of stable types to be changed to the newer designation in the 'replace stable type' step

| Old | New |
| --- | --- |
| BATT_OV | KOOI_OV |
| GRONDH_OV | NKOOI_OV |
| GRONDH_NEA | NKOOI_NEA |
| RUN_NEVER | NO |

### Delete records with empty animal subcategory codes

Records lacking the animal subcategory code in the input tables DIER_PROD (contains gross nitrogen and phosphorus production per animal subcategory) and STALLEN (= stables) are deleted.

### Replace historical animal subcategory codes

The historical animal subcategory codes 242 and 243 (pigs 2- and 3-phase feeding regime) are replaced by code 124 (Other pigs from 20 to 110kg). If this replacement creates several stables per exploitation with the same combination of animal subcategory and stable type, these stables will be combined (see Combine stables).

Figure 2: Overview of the decision rules followed when assigning a stable type in the 'replace stable type' step to records where this is missing from the input data

### Replace EA stables

For the years 2007 to 2013, the action Replace EA stables is implemented. This editing of the input data is necessary because the VLM input data for those years is incomplete in terms of the use of Low Ammonia Emissions ) Stable Systems. This editing step means that for certain combinations of exploitations and animal subcategories, the stable systems in the input table STALLEN are replaced by the data included in two .csv files containing the missing data. The animal subcategories in question are: 121, 122, 123, 124, 125, 131, 132, 133, 134, 98.

In the EMAV 2.1 model, this step had to be done manually by the user, in EMAV3.0 it is done automatically when loading the data.

### Combine stables

When multiple records with the same combination of animal subcategory and stable type occur in the input table STALLEN for a given exploitation, they are combined into one record. The animal density and gross nitrogen and phosphorus production of the different records are added together. The original records are removed from the input table STALLEN. Consequently, for each combination of exploitation, animal subcategory and stable type, there is at most one record present in the input table STALLEN.

### Verify input

After the user has loaded a dataset and has chosen a calculation factor set, a verification step must be performed. Whenever the year of the loaded dataset does not match the year in the selected calculation factor set, the user is notified by means of the pop-up shown in Figure 3.

Figure 3: Pop-up that appears when the user wants to start a calculation for a dataset whose year differs from the year of the selected calculation factor set.

The Verify input functionality performs some checks on the selected input data and calculation factor set. These checks are listed below:

- Report empty tables

- Report columns indicated in tblCheck without values

- Report columns indicated in tblCheck with negative values

- Report columns indicated in tblCheck whose sum is 0

- Check that all animal subcategories in the input tables DIER_PROD and STALLEN appear in the calculation factor table tblDierSubCategorie.

- Check which combinations of animal subcategories and stable types do not occur exactly once in the calculation factor table lnkStalDierSubCategorieNH3.

- Check which combinations of animal subcategories and stable types occur multiple times per exploitation in the input table STALLEN. (This should not normally be allowed as these combinations are taken together in the Clean-up input data step as explained above).

- Check that all PAS measures (Programmatic Approach to Nitrogen) from the input table AANGIFTE_DIER_STAL_PAS are found in the calculation factor table tblPAS.

- Check that all stable types from the input table AANGIFTE_DIER_STAL_PAS are found in the calculation factor table lnkStalDierSubCategorieNH3.

- Check that all animal subcategories from the input table AANGIFTE_DIER_STAL_PAS are found in the calculation factor table tblDierSubCategorie.

- Check for which exploitations and animal subcategories there are PAS measures and multiple stable types.

- Check for which combinations of exploitations, animal subcategories and livestock types the animal numbers from the input table AANGIFTE_DIER_STAL_PAS exceed those from the input table STALLEN.

- Check for which combinations of exploitations and animal subcategories from the input table AANGIFTE_DIER_STAL_PAS the stable types do not occur in the input table STALLEN.

- Check which manure codes from the transport input tables are not found in the input table CO_SLIB.

- Check whether all combinations of animal subcategories and manure types appear in the calculation factor tables tblStal3 and lnkStalAnimalSubCategorieNH3

- Check which crops from the input table BEMESTING are not found in the calculation factor table lnkGEWASSEN.

- Check which fertiliser codes from the input table GEBRUIK_KM are not found in the calculation factor table lnkKUNSTMEST.

Once the checks have been performed, a pop-up is displayed showing the result of the checks (see Figure 4 as an example). If desired, this pop-up can be written to the folder where the results of the calculations end up. The information from the pop-up allows the user to make adjustments to the calculation factor sets if necessary (see Figure 5 for an example pop-up requiring action). This is the case, for example, if a new stable system or crop code appears in the input data. This must then be added to the calculation factor set. If the pop-up shows errors in the input data (e.g. empty columns), a new dataset should be requested from the Manure Bank. It requires a skilled user to estimate if and where action is needed.

Figure 4: Example pop-up after verify input with no error messages requiring action to do the calculation. The named empty tables do not affect the calculations.

Figure 5: Example pop-up after verify input with error messages requiring action in order to perform a calculation. The absence of the listed measures in the PAS calculation factors table prevents a proper calculation of ammonia emissions from the stable.

# Calculating emissions

## Nitrogen excretion and nitrogen flow

The calculation of nitrogen emissions in section 2 (Manure Management) and section 3 (Agricultural Soils) is based on the nitrogen production per animal subcategory and type of stable. These data are available from the year 2007 onwards per agricultural exploitation, and is provided by the Manure Bank.

EMAV follows the nitrogen flow through the farm. In the first step, nitrogen production from the input data is divided between the emission stage Pasture and the emission stage Stable. This distribution is done on the basis of grazing time. This grazing time is calculated generically for the first part of the time series, 1990-2015, via a fixed number of grazing days per animal subcategory. From 2016 onwards, the input dataset contains exploitation-specific data on grazing time. These data are not available for earlier years.

At the emission stage Stable, nitrogen emissions (NH3, N2O, NOx and N2) are calculated and then subtracted from the part of nitrogen production allocated to the stable (after taking into account grazing time). Any nitrogen remaining in the manure according to this calculation then passes to the emission stage Storage. In EMAV3.0, there is an exception to this in case a pocket digester is present on the exploitation. In that case, the liquid cattle and swine manure does not go to external storage, but to the pocket digester. It is assumed that other manure types do not go to the pocket digester. Both in the external storage and in this small-scale manure treatment, nitrogen losses are calculated on the amount of nitrogen supplied.

After ES storage, manure transport data is used to calculate the removal of manure from storage. Part of the manure will be transported to a manure processing plant and gives rise to emissions in the Manure Processing emission stage. The part of the manure that remains after taking into account the manure transports is distributed over the land in the emission stage Land application (section 3 - Agricultural soils).

## Volatile solids-excretion

Methane emissions are calculated in EMAV3.0 from the amount of volatile solids (VS) excreted by each animal (sub)category. VS excretion is not provided through the input data, but is calculated in EMAV3.0 for each animal subcategory.

For cattle, a Tier 2 methodology, based on the IPCC 2019 Refinement Guidelines for National Greenhouse Gas Inventories (IPCC, 2019), is used for this purpose. The calculation is based on gross energy intake and feed digestibility. For pigs, a region-specific methodology is used (Tier 3), which takes into account daily feed intake by subcategory, digestibility, dry matter content and ash content of the feed. The same values are used for the entire time series. For the remaining animal categories, the VS excretion per animal (sub)category is calculated according to the IPCC 2019 Tier 1 methodology. Here, default excretion values are available, valid for the full time series.

Similar to nitrogen excretion, volatile solids excretion is divided between the emission stages Pasture and Stable based on grazing time.

## Section 1: Enteric emissions

### Emission stage 'Enteric emissions'

The emission stage 'Enteric emissions' computes methane emissions (CH4) generated during the digestion process in the rumen and large intestine in farm animals.

The methodology for calculating enteric emissions is based on IPCC 2019. Here, total enteric methane emissions are calculated by multiplying an emission factor (EF) per animal (sub)category by the number of animals in the animal (sub)category.

The calculation of the emission stage 'Enteric emissions' is done for each animal subcategory at the level of the agricultural exploitation. Depending on the size of the emissions, a Tier 1 or Tier 2 methodology is used to determine the emission factor. Cattle is a key source category for CH4 emissions from enteric fermentation in Flanders, as these emissions have a significant impact on total Flemish greenhouse gas emissions. Therefore, a more detailed Tier 2 methodology is required. For the non-key source categories pigs, ostriches, sheep, goats and horses, a less detailed Tier 1 methodology is applied.

Enteric emissions from fur animals (mink) and rabbits are not calculated given the low number of animals in Flanders and the fact that there is no default emission factor available in IPCC 2019 guidelines. There is also no default IPCC emission factor available for chickens and turkeys (IPCC, 2019) and the contribution of fermentation to feed digestion is considered negligible.

### Tier 1 methodology

For animal categories with Tier 1 methodology (IPCC, 2019), the total number of animals of a given animal category is multiplied by the default emission factor from the IPCC 2019 guidelines. The default emission factors are assumed to remain the same over the full time series.

For pigs, sheep and goats, the IPCC 2019 guidelines distinguish between low and high-yielding systems. For Flanders, the emission factors for high production apply.

For sheep and goats, the EMAV3.0 input dataset taken from the Manure Bank distinguishes between animals younger and older than 1 year. For animals older than 1 year, the default emission factor from the IPCC 2019 guidelines is applied. For animals younger than 1 year, a specific emission factor was calculated according to the methodology of Rösemann et al. (2017).

### Tier 2 methodology

The Tier 2 methodology (IPCC, 2019) does not use default emission factors, but calculates an EF per cattle subcategory based on the gross energy intake (GE) and methane conversion factor

(Ym ) of each subcategory.

#### Gross energy intake (GE)

Gross energy intake is calculated stepwise with the digestible energy of the ration and the energy required for body maintenance, activity, lactation, pregnancy and growth.

#### Digestible energy (DE)

The digestible energy of the average rations is calculated for each cattle subcategory.

Region-specific average rations for dairy cattle and brood cows were established based on literature data (Curial et al. 2018). This distinguished between summer and winter rations. For the other subcategories, it is not possible to specify the ration due to the lack of sufficient data, and therefore a default digestibility is calculated for the ration.

For dairy cows, slaughter calves and brood cows, the ration composition changes over the time series.

#### Net energy for body maintenance (NEm)

Net energy for body maintenance is calculated using a body maintenance coefficient (Cfi) and average live body weight (BW). For the Cfi, the default values from the IPCC 2019 guidelines are used. The values remain constant over the time series. For animal weights, region-specific values are used which, due to lack of more info, do not vary over the time series.

#### Net energy for growth (NEg)

In the next step, the NE required for growth is calculated. The applied coefficient for growth (C) is based on the IPCC 2019 default values, namely 0.8 for females and 1.2 for bulls. The applied average live body weight (BW), mature weight (MW) and average daily weight gain (WG) of animals are region-specific. Values for BW, MW and WG remain constant over the time series.

#### Net energy for activity (NEa)

The NE for activity is calculated using an activity coefficient (Ca ) and the NE for maintenance (NEm).

The calculated Ca coefficients are based on the default values from IPCC 2019. For animals kept in stables, the default Ca equals 0. Animals grazing on a pasture need an amount of energy to absorb feed. This required energy is calculated by multiplying the default Ca ( 0.17) by the % of time the animal spends on pasture.

For the 1990-2015 time series, a fixed number of grazing days per animal subcategory are used, which were determined based on expert judgement. From 2016, exploitation-specific values are available in the input dataset. These reflect the % of time animals spend in stables per animal subcategory and stable type.

#### Net energy for lactation (NEl)

The NE for lactation depends on the amount of milk (kg/day) produced on average and the fat content of the milk (%).

Milk production data for dairy cows in the 1990-2006 time series are not known for milk yields at farm level. Therefore, an average milk production at the Flanders level is used. From 2007, the milk yield of dairy cows at exploitation level can be derived from the input dataset of the Manure Bank.

For brood cows, milk yield decreases linearly from 4.66 kg milk/day in 1990 to 0.72 kg milk/day in 2018. From 2018, milk yield is kept constant at 0.72 kg milk/day.

The fat percentage of milk (fat) is requested annually from the Agency for Agriculture and Fisheries.

#### Net energy for pregnancy (NEp)

Net energy for gestation is calculated only for the animal subcategories dairy cows and brood cows. This is calculated using a coefficient for pregnancy (Cpregnancy) and the NE requirement for maintenance (NEm). The coefficient for pregnancy is equal to the default value of 0.10 for dairy and brood cows (IPCC, 2019). In addition, 80% of the subcategory is assumed to be pregnant during a calendar year (cf. IPCC Guidelines for National Greenhouse Gas Inventories).

#### Methane conversion factor (Ym)

The fraction of gross energy in the ration that is converted to methane is reflected by the methane conversion factor (Ym). Region-specific figures are used, based on rations representative of the Flemish situation.

The methane conversion factor is 6.1% for all categories of cattle and remains constant throughout the time series, except for slaughter calves. The Ym for dairy cattle was determined based on data from the SMETHANE project. In the absence of specific data for the other cattle subcategories, the Ym used for dairy cattle was adopted for the other subcategories.

A Ym of 0% is used for the subcategory slaughter calves until 2006 and from 2007 the Ym is 0.854%. It is assumed that up to 2006 the ration consists of 100% milk replacer and that from 2007 the ration consists of 86% milk replacer and 14% roughage. Only the roughage leads to the production of enteric methane emissions. Considering a Ym of 6.1 for 14% of the ration, a Ym for slaughter calves of 0.854% is obtained.

### Reduction measures Covenant Enteric Emissions Cattle

In 2019, the Flemish Government entered into a covenant 'Enteric Emissions Cattle 2019-2030' (CEER) with the sector to reduce enteric methane emissions from cattle, for instance by adjusting feed rations. These feed measures came into effect in 2022. The emission reductions obtained by adjusting the feed ration can be calculated in EMAV3.0.

The data of farms applying feed measures are obtained via the Annual Farm Inventory of the Agency of Agriculture and Fisheries. Based on the period of the year in which the feed is provided to a certain animal (sub)category, the animal numbers and the reduction percentage of the relevant feed measure, a reduction on enteric methane emission is applied in EMAV3.0.

## Section 2: Manure management

### Emission stage 'Pasture'

Ammonia emissions on pasture are calculated by multiplying nitrogen production on pasture by a region-specific emission coefficient (= 8%).

Methane emissions from grazing are calculated following the IPCC 2019 Tier 2 method where volatile solids excreted on pasture are multiplied by the IPCC 2019 default methane conversion factor for grazing and a default maximum methane production capacity (B0). These values are valid for all animal subcategories and for the entire time series.

For the methodology of N2O and NOx emissions from grazing, see Section 3 - Agricultural soils.

### Emission stages ‘Stable’ and 'Storage'

In EMAV3.0, stable and external manure storage are considered as two separate emission stages when calculating nitrogen losses of NH3. Emissions of N2O, NOx and N2 are calculated for the whole of stable and external storage together. The calculation is done per stable type and per animal subcategory.

Unlike the other nitrogen emissions, stable ammonia emissions are calculated based on animal numbers and region-specific emission factors. Consequently, this is not based on the amount of manure produced in the stable. The possibility is provided to calculate emission reductions through grazing or emission-reducing measures, e.g. air scrubbers.

To calculate other nitrogen losses (N2O, NOx and N2) and methane emissions, stall systems from the Manure Bank data were linked to an IPCC manure management system. This distinguishes between solid and liquid manure.

Other nitrogen losses are calculated based on the gross nitrogen production in the stable minus the nitrogen volatilised as NH3. For nitrous oxide, the default IPCC 2019 emission factors of the different manure management systems are applied. The emission factors of NOx are set equal to those of N2O. For N2, the emission factor for solid manure is 5 times the EF of N2O, and for liquid manure 10 times the EF of N2O (method according to Oenema et al. 2000). These emission factors apply to the entire time series. The calculation of nitrogen losses from N2O and NOx also includes the possibility of accounting for emission reductions due to stable techniques or grazing.

These nitrogen losses are then subtracted from the 'Gross production without ammonia' to arrive at the 'Net production in the stable'. This amount of nitrogen then flows to external storage or possibly to a pocket digester.

NH3 emissions from external storage are also calculated. The region-specific emission factors are based on the ECOLAS study on external manure storage (2006). The IPCC 2019 emission factors for N2O from stables already account for external storage. Therefore, N2O emissions are not calculated separately at this stage, nor are NOx and N2 emissions, as per Oenema et al. (2000).

Indirect nitrous oxide emissions are calculated using the emitted ammonia and nitrogen oxides from the stable and storage. Here, the sum of ammonia and nitrogen oxide emissions is taken and multiplied by the default emission factor for wet climate from IPCC 2019.

Methane emissions are calculated in EMAV3.0 for the whole of stable and external storage together. The default emission factors as stated in the IPCC 2019 guidelines are provided for one manure management system (MMS) that includes both the stable and storage system. Emissions are calculated separately for the liquid and solid fraction.

Methane emissions from manure management are calculated by multiplying the excreted volatile solids per animal subcategory and per manure management system by the maximum methane production capacity (B0) of the animal subcategory and the methane conversion factor (MCF) of the manure management system.

In the IPCC 2019 guidelines, different methane conversion factors are given for a number of manure management systems as a function of manure storage duration. As no data on effective storage duration are available for Flanders, a retention time of 6 months is used by default, as recommended by the IPCC. The exception is liquid cattle and swine manure that flows to a pocket digester. This manure is only stored for a short time and for this the default MCF for a retention time of 1 month is used. The MCF’s corresponding to a Cool Temperate Moist Climate are used because the average winter temperature (storage occurs mainly during this period) in Belgium is well below 10°C.

The maximum methane production capacity for cattle and pig manure are region-specific; for the other animal categories, the value from the IPCC guidelines (IPCC, 2019) was adopted.

As with the methodology for nitrogen losses, the formulas for methane emissions also include the possibility of calculating emission reductions in the future through stable techniques or grazing.

### Emission stage 'Small-scale manure processing'

Small-scale manure processing includes pocket digestion at farm level, where the exploitation’s own manure is converted to biogas and digestate. The digestate remaining after pocket digestion is not further used in the model for the time being due to lack of data.

From 2012, data are available in the Manure Bank dataset on the presence of pocket digesters on agricultural farms. However, no data are available on the amount and type of manure that is effectively digested. It is therefore assumed that all liquid cattle and swine manure on the exploitation goes to the pocket digester when a pocket digester is present on that exploitation. Manure from other animal categories is assumed not to be processed in the pocket digester.

Nitrogen emissions that occur during small-scale manure treatment are calculated by multiplying the amount of nitrogen digested per animal subcategory by the emission factor of the relevant pollutant. In the absence of specific emission factors for pocket digestion, the same emission factors as for digestion at professional manure processors are used.

Methane emissions are calculated by multiplying the amount of volatile solids digested on the farm by the maximum methane production capacity of the manure belonging to the animal subcategory and the methane conversion factor (MCF) for digestion. For anaerobic digestion, different MCFs are available in IPCC 2019 based on technology quality (leakage losses) and digestate storage. For pocket digestion, the value for 'Anaerobic digester, Low leakage, High quality industrial technology, low quality gastight storage technology' was adopted. The default B0 values are corrected for methane losses occurring in the stable and storage.

### Emission stage 'Manure processing'

In Flanders, a manure treatment obligation has been in place since 2000 with the entry into force of the Manure Action Plan (MAP) II bis in 2000.

The emission stage 'Manure processing' calculates the emissions of ammonia and nitrous oxide, from professional manure processing installations from the production year 2008 onwards. The calculation takes into account all animal manure arriving at these manure-processing plants, not just manure from Flemish agricultural exploitations. These manure quantities are determined on the basis of animal nitrogen transports reported to the Manure Bank. 2008 is the oldest year for which these transport data are considered complete enough to estimate emissions caused by manure processing.

One main processing technique is assigned to each manure processing plant from a priority list, namely 1. biology, 2. composting, 3. digestion, 4. Physico-chemistry or drying.

Based on the manure transfers known at the Manure Bank, the N quantity supplied to the approved manure processors is determined. Emissions of NH3 and NOx are calculated by multiplying the amount of nitrogen processed on the farm by an emission factor or coefficient corresponding to the processing technique used.

Emission coefficients for NH3 were obtained from the literature. The emission factors for NOx were, according to the methodology of Oenema et al (2000), equated to the N2O emissions factors from the IPCC 2019 guidelines for the respective manure management systems (composting = 'static pile (forced aeration)'; digestion = 'anaerobic digester' and biology = 'aerobic treatment- forced aeration systems'). No NOx emissions are calculated for the manure treatment techniques 'physico-chemistry' and 'drying', due to the lack of emission factors.

## Section 3: Agricultural soils

The section Agricultural Soils covers emissions of ammonia, nitrous oxide and nitrogen oxides from agricultural soils in Flanders. The emissions are calculated per year at plot level and depend mainly on the amounts of manure applied to the plot in the given year. The necessary information about the plots, such as surface area and crops, are obtained via the Manure Bank. The Manure Bank receives the data via the ‘Collective Request’ that farmers have to fill in annually.

The emissions at Flanders level are calculated by summing the emissions calculated at plot level.

### Emission stage ‘Pasture’

The calculation of ammonia and methane emissions from grazing is explained in section 2 - Manure management.

Direct nitrous oxide emissions on pasture are calculated according to the IPCC 2019 Tier 1 method. This involves multiplying the amount of nitrogen applied to pasture by grazing animals by default IPCC 2019 emission factors per animal category. Emissions of nitrogen oxides are calculated by multiplying this amount of nitrogen by the default emission factor from the European Monitoring and Evaluation Programme (EMEP 2019).

### Emission stage 'Land application'

### Animal organic manure

A region-specific methodology is used to distribute the available manure per exploitation among the agricultural plots belonging to this exploitation. The available nitrogen for spreading is determined on the basis of the nitrogen quantities that, according to the calculations, are still present in the manure (per animal category and manure type (= solid/liquid)) after going through the emission stages of stable, storage and/or small-scale manure treatment and after taking into account the transports reported to the Manure Bank. 2008 is the oldest year for which these transport data are considered complete enough to estimate emissions caused by the application of animal organic manure.

Per animal category and manure type, the available nitrogen is ranked (= priority for spreading). The plots are also ranked (= priority for receiving nitrogen) based on the main crops on the plot. The main crops are grouped into the following crop groups: potatoes, beets, fruit trees, cereals, grass, green manures, vegetables, maize, leguminous crops, other crops and no fertiliser. Manure distribution starts with the nitrogen group and plots with the highest rank order and fills these plots up to the legally imposed maximum amount of nitrogen from livestock manure. The manure already allocated to the plot by grazing, as calculated in the emission stage Pasture, is taken into account while filling up to the legal maximum. Certain combinations, such as solid manure on grassland, are not allowed. If there is still nitrogen left on the exploitation, after going over all the plots, then further fertiliser is applied up to 150% of the legal maximum for animal manure. At the Flanders level, this local over-fertilisation is compensated by not (or not fully) fertilising other plots due to a mismatch between available manure and crops. Manure remaining on the farm is then added to the surplus from other farms and distributed evenly over all Flemish plots from all farms to which animal manure was allocated.

### Synthetic fertiliser

Per exploitation, the available amount of synthetic fertiliser is divided over the plots, starting with the plots with the least amount of animal manure and taking into account the plot standard for active nitrogen. Any nitrogen remaining after all plots have been covered, will be evenly distributed over all Flemish plots belonging to exploitations which have declared synthetic fertiliser use for the given year.

The fertiliser quantities used per exploitation are taken from the Manure Bank. A correction factor (varying annually) is applied to these quantities as it is assumed that the Manure Bank data give a general underestimation of the total use of synthetic fertiliser in Flanders. This correction factor for synthetic fertiliser use is calculated annually by the Agency for Agriculture and Fisheries and is derived from data obtained via the agricultural monitoring network.

### Emission calculation

Direct nitrous oxide emissions are calculated according to the IPCC 2019 Tier 2 methodology. Both formulas and default emission coefficients were adopted. Nitrogen from grazing of agricultural plots, use of organic soils and different nitrogen inputs per plot are taken into account. Nitrogen inputs include synthetic fertiliser applied to agricultural plots, applied organic manure, crop residues left after harvest and mineralisation of soil organic matter in mineral soils. Data on organic soils are available only at the municipality level. Each agricultural plot in the municipality is therefore assigned the same proportion of organic soil, the remaining part of the plot is then considered mineral soil. The same default emission factor is used for all fertilisation techniques, due to the lack of more differentiated emission factors.

Emissions of nitrogen oxides are calculated by multiplying the nitrogen amounts on the plot from grazing and application of organic manure and fertiliser by the default EMEP2019 emission factor.

To calculate ammonia losses, the methodology from EMAV2.1 is used. For each application site (cropland, pasture, orchard), the common techniques for applying livestock manure, their share of use and the corresponding emission coefficients for ammonia are taken into account. The share of use of the different manure application techniques is set at Flanders level, since there are no data on the techniques used at the farm level. As the emission coefficients are expressed as a percentage of total ammoniacal nitrogen (TAN), the share of TAN in the nitrogen quantities distributed to the fields is first calculated. After which the emissions are calculated. For synthetic fertiliser, the nitrogen quantities on the plots are multiplied by the emission coefficient belonging to the synthetic fertiliser group.

### Emission stage ‘Crop residues’

The nitrogen input on Flemish agricultural plots from crop residues left behind after crop harvesting is calculated in EMAV3.0 according to the IPCC 2019 Tier 1 methodology and this from the year 2007 onwards. For the older years, necessary plot-level data are missing to perform the calculations. Both the crop residues of the main cultivation and cultivations on the same plot in the same year before and after this main cultivation are taken into account. Cultivation areas are extracted from the input data from the Manure Bank. The total nitrogen amount in the above- and belowground crop parts that remain on the field after the harvest is calculated using region-specific yield data (from Statbel) per crop group and default calculation values for e.g. dry matter content (IPCC2019). For grain crops, the removal of part of the crop residue for use as straw is also taken into account (FRAC_REMOVE = 0.9). Burning of crop residues is not considered relevant in Flanders and thus not included in the calculation.

Both direct nitrous oxides and ammonia emissions are calculated in EMAV3.0 at the emission stage crop residues, using the default emission factors from IPCC 2019 and the 2023 EMEP Guidebook, resp.

### Emission stage 'Soil type'

Direct nitrous oxide emissions caused by the mineralisation of soil organic matter are calculated according to IPCC 2019 Tier 1 methodology. The calculation of this nitrogen input uses the carbon loss from mineral soils calculated annually by the emission inventory air of Flanders Environment Agency (VMM), the ratio of carbon to nitrogen in soil organic matter in Flanders and the default IPCC 2019 emission factor.

Direct nitrous oxide emissions from managed organic soils are calculated by multiplying the annual area of managed/drained organic soils by the emission factor per plot (methodology from IPCC 2014 Supplement Report). This area of organic soils was estimated in 2001, as part of the BELSPO-METAGE research project, by the Research Institute for Nature and Forest (INBO) in collaboration with the Catholic University of Leuven (KU Leuven). For all pasture plots in Flanders, the emission factor for nutrient-rich and deeply drained pasture is used.

### Indirect nitrous oxide emissions

Indirect nitrous oxide emissions are calculated in EMAV3.0 according to an IPCC 2019 Tier 2 methodology. These emissions result from ammonia emissions and nitrogen oxide emissions from agricultural plots. A distinction is made between emissions due to nitrogen deposition and emissions due to nitrogen leaching and runoff.

Indirect nitrous oxide emissions from the deposition of nitrogen volatilised from managed soils are calculated based on the amount of ammonia and nitrogen oxides released from the use of synthetic fertiliser and organic manure as well as from grazing. This nitrogen amount is then multiplied by the default IPCC 2019 emission factor.

Indirect nitrous oxide emissions caused by leaching and runoff from applied nitrogen are calculated using the IPCC 2019 default values for the leaching fraction and emission factor. Leaching and runoff are assumed to occur throughout the whole of Flanders.

# Geographical distribution of emissions

The user has the choice of running the emission calculations with or without geographical distribution. If the choice is made to calculate with geographical distribution, coordinates will be added to the calculated emissions for the emission stages ES Enteric Emissions, ES Stable, ES Storage and ES Small-scale manure treatment. The emissions calculated in these emission stages will all be linked to the coordinates of the specified stable plots. At present, the input dataset does not yet contain data allowing the specified stable types to be linked to the stable plots, so the following calculation rules are applied:

- Multiple stable plots, but all at maximum 100 m from the centroid between the stable plots -> emissions are allocated to the centre of gravity

- Multiple stable plots, but at least 1 at more than 100 m from the centroid between the stable plots -> the emissions are distributed equally between the different plots

If for a specific exploitation no stable plots (and thus no stable plot coordinates) can be found in the input dataset, the coordinates of the administrative address of this exploitation are used.

The agricultural plot table in the input dataset contains coordinates for all agricultural plots in Flanders, not just the stable plots. Therefore, coordinates are added to all emission calculations that use these input data. These are calculations for ES Pasture, ES Land application, ES Crop residues, ES Soil Type. Also for ES Manure Processing, coordinates per exploitation are already available in the input dataset, so they are linked to the calculated emissions by default.

# Output tables

When calculating with EMAV3.0, a folder is created with several .xlsx files containing the calculated emissions per emission stage. Both tables at the smallest level of calculation, for example plot level, animal subcategory, stable type... and more aggregated tables are created. The detailed tables, together with the export of the calculation factors that is also saved in the same output folder, contain all the information needed to check the calculations. In addition, output files are created in the required format for (annual) reporting to various target groups, including the United Nations and the European Commission as part of the mandatory international reporting of air pollutant and greenhouse gas emissions.

# Interpretation of results

EMAV3.0 was constructed with the aim of providing the best possible estimate of non-energy emissions of greenhouse gases, ammonia and nitrogen oxide caused by Flemish agriculture as a whole. In all circumstances, the obtained results should be interpreted with caution. All data used in the calculations is subject to some degree of uncertainty. Therefore it must always be taken into account that although a very detailed data set is used, the more one deviates from this Flemish level the greater the uncertainty on the results becomes. This is partly due to emission factors that should rather be considered as calculation factors that try to reflect the situation of 'the average Flemish farm' or 'the average Flemish agricultural soil' than a calculation value that applies to each individual stable or each individual agricultural plot.

# References

Curial, S., De Boever, J., De Campeneere, S., De Cuyper, C., Delezie, E., Goosens, K., Millet, S., Molnar, A., Vandaele, L. 2018. Mestproductie reduceren via voedermaatregelen: stikstof- en fosfaatexcretie reduceren via voedermaatregelen bij rundvee, varkens en pluimvee in de context van de maximale actielijst ter verbetering van de waterkwaliteit, Maatschappij, V.L., ed.

ECOLAS. (2006): Externe mestopslag: inventarisatie van opslagsystemen en bepaling van ammoniak-, lachgas- en methaanemissies uit deze systemen. Studie uitgevoerd door Ecolas en UGent in opdracht van LNE, Afdeling Lucht, Hinder, Risicobeheer, Milieu & Gezondheid en Departement Landbouw en Visserij, Afdeling Monitoring & Studie.

Oenema, O., Velthof, G. L., Verdoes, N., Groot Koerkamp, P. W. G., Monteny, G. J., Bannink, A., Van der Meer, H. G., & Van der Hoek, K. W. (2000). Forfaitaire waarden voor gasvormige stikstofverliezen uit stallen en mestopslagen.

Rösemann, C., Haenel, H-D, Dammgen, U., Freibauer, A., Doring, U., Wulf, S., Eurich-Menden, B., Dohler, H., Schreiner, C., Osterburg, B. (2017). Calculations of gaseous and particulate emissions from German Agriculture 1990 – 2015. Thunen Report 46, pp 424.

Content details on this covenant can be accessed at the following link: https://www.rundveeloket.be/CEER/maatregelen

More information on the agricultural monitoring network can be accessed at the following link: https://landbouwcijfers.vlaanderen.be/landbouwmonitoringsnetwerk